| BRAND | PLATATAC |
| --- | --- |
| SIZE RANGE | 2XS-3XL (REG, SHORT& LONG) |
| BASE SIZE | MEDIUM |
| MAIN FABRIC | NYCO RIPSTOP, POLARTEC |

BACK VIEW

FRONT VIEW

COLLAR, SLEEVE, POCKET; NYCO RIPSTOP

BODY; POLARTEC

COMMENTS: BODY FABRIC CAN BE CHANGE FOR SOME COLOR DEPENDS ON CUSTOMER REQUEST

ZIP PULL CORD

REVERSABLEL ZIP

FRONT ZIP OPENING;

#3 ZIP REVERSABLE C/E

ELBOW POCKET OPENING TO INSERT ELBOW PAD;

20MM VELCRO - HOOK & PILE

ELBOW POCKET

SLEEVE POCKET;

SL FLAP TOP - 50MM PILE VELCRO

POCKET- 38MM PILE VELCRO

UNDER FLAP; 25MM HOOK VELCRO

POCKET TOP; 25MM PILE VELCRO

BUTTONHOLE

SLEEVE FLAP (TOP)- PEN POCKET OPENING 2.5CM

SLEEVE POCKET ZIP;   #3 NORMAL ZIP

ZIP PULL CORD

SLEEVE CUFF (VELCRO ADJUSTABEL);

SLEEVE CUFF- 25MM PILE  VELCRO

SLEEVE TAB- 20MM HOOK VELCRO

RM Color Chart

| FABRIC COLOR | BODY (POLARTEC) | SLEEVE& COLLAR (NYCO RIP STOP) | FRONT ZIP #3 REVERABLE ZIP | SLEEVE PKT- #3 NORMAL ZIP | ZIP PULL CORD- FRT & SL PKT | 25MM VELCRO PILE -SL PKT | 20MM VELCRO HOOK & PILE – ELBOW PKT, SL CUFF TAB, SL POCKET | 25 MM PILE VELCRO – SLEEVE CUFF | 38MM &50MM PILE VELCRO SL PKT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MULTICAM | TAN 499 | MC | YKK 886 | YKK 886 | DTM TO ZIP | KHAKI | KHAKI | MULTICAM | MULTICAM |
| MC TROPIC | RG | MC TROPIC | YKK 096 | YKK 096 | RG | RG | RG | RG | RG |
| MC BLACK | BLACK | MC BLACK | YKK 580 | YKK 580 | BLACK | BLACK | BLACK | MC BLACK | MC BLACK |
| BLACK | BLACK | BLACK | YKK 580 | YKK 580 | BLACK | BLACK | BLACK | BLACK | BLACK |
| RANGER GREEN | RG | RG | YKK 096 | YKK 096 | RG | RG | RG | RG | RG |
| MC ARID | TAN 499 | MC ARID | YKK 886 | YKK 886 | DTM TO ZIP | KHAKI | KHAKI | MC ARID | MC ARID |
| AOR 2 | RG | AOR 2 | YKK 096 | YKK 096 | RG | RG | RG | AOR 2 | AOR 2 |
| FROG SKIN | RG | FROG SKIN | YKK 096 | YKK 096 | RG | RG | RG | RG | RG |
| ARO 1 | CBK | ARO 1 | YKK 219 | YKK 219 | CBK | CBK | CBK | CBK | CBK |
| M90 | RG | M90 | YKK 096 | YKK 096 | RG | RG | RG | RG | RG |
| ROYAL THAI ARMY | ROYAL THAI ARMY (BOSS RUGBY) | ROYAL THAI ARMY | YKK 580 | YKK 580 | BLACK | RG | RG | RG | RG |
| TFP | BROWN (BOSS RUGBY) | TFP | #: 394 | #: 394 | MATCH TO ZIP | KHAKI | KHAKI | TFP | TFP |
| NAVY BLUE | NAVY BLUE | NAVY BLUE | YKK 169 | YKK 169 | NAVY BLUE | NAVY BLUE | NAVY BLUE | NAVY BLUE | NAVY BLUE |
| WOODLAND | RG | WOODLAND | YKK 096 | YKK 096 | RG | RG | RG | WOODLAND | WOODLAND |
| 3 COLOR DESERT | CBK | 3 COLOR DESERT | YKK 219 | YKK 219 | CBK | CBK | CBK | CBK | CBK |
| TIGER STRIPE | BLACK | TIGER STRIPE | YKK 580 | YKK 580 | BLACK | BLACK | BLACK | BLACK | BLACK |
| DTS | CBK | DTS | YKK 219 | YKK 219 | CBK | CBK | CBK | CBK | CBK |

LABEL & PACKING

HEAT SEAL LABEL:

PLACEMENT- Back panel 2cm from neck seam & centered

PRINT SIZE:

HIGHT 5CM X WIDTH 6.5CM

| FABRIC COLOR | PRINT COLOR |  | FABRIC COLOR | PRINT COLOR |
| --- | --- | --- | --- | --- |
| MC | White |  | M90 | White |
| MC TROPIC | White |  | ROYAL THAI ARMY | Black |
| MC BLACK | White |  | TFP | White |
| BLACK | White |  | NAVY BLUE | White |
| RG | White |  | WOODLAND | White |
| MC ARID | White |  | TIGER STRIPE | White |
| AOR 1 | White |  | DTS | White |
| AOR 2 | White |  | 3 COLOR DESERT | White |
| FROG SKIN | White |  |  |  |

| FABRIC COLOR | WOVEN LABEL COLOR |  | FABRIC COLOR | WOVEN LABEL COLOR |
| --- | --- | --- | --- | --- |
| MC | Khaki |  | M90 | RG |
| MC TROPIC | RG |  | ROYAL THAI ARMY | RG |
| MC BLACK | Black |  | TFP | Khaki |
| BLACK | Black |  | NAVY BLUE | Black |
| RG | RG |  | WOODLAND | RG |
| MC ARID | Khaki |  |  |  |
| AOR 2 | RG |  |  |  |
| FROG SKIN | RG |  |  |  |
| ARO 1 | Khaki |  |  |  |

WOVEN LABEL: WEAR’S LEFT SLEEVE POCKET ATTACH 2.5CM FROM BOTTOM

DATE LABEL – PVC

0.7CM – SEWING ALLOWANCE

2.7CM

DOM

04/2026

DATE LABEL PLACEMENT

8CM FROM BOTTOM HEM - WLS

2.5CM

HANG TAG: PLATATAC LARGE HANG TAG

TAG SIZE; HIGHT 12CM X WIDTH 5CM

HANG TAG; On one side Platatac log & another side sticker. Tie with string to zip slider.

PACKING; Shirt pack Individually in PLATATAC poly bag with polybag sticker – POLYBAG SIZE = 40CM x 48CM

STICKER ARTWORK – HANG TAG AND POLYBAG

STICKER SIZE: HIGHT 4CM X WIDTH 9CM

SPEC SHEET

Garment Specification – TOPS 17/04/2024. STYLE NO:; Cuts shirt V4 (NEW); CUSTOMER:; Platatac. FABRIC:; Woven /Polartec; SIZE RANGE:; 2XS - 3XL; BASE SIZE:; M. BLOCK:; DIGITISED PATTERN. GARMENT DETAILS:; 17/09/22. FINISHED GARMENT MEASURMENTS; 2XS; XS; S; M; L; XL; 2XL; 3XL; TOL - /+; GRADE. CHEST CIRCUM. ARMHOLE; 82; 88; 94; 100; 106; 112; 118; 114; 2; 6. HEM CIRCUMFERENCE; 78; 84; 90; 96; 102; 108; 114; 110; 2; 6. BACK LENGTH SHORT (NECK SEAM TO HEM); 75; 76; 77; 78; 79; 80; 81; 82; 1; 1. BACK LENGTH REGULAR (NECK SEAM TO HEM); 79; 80; 81; 82; 83; 84; 85; 86; 1; 1. BACK LENGTH LONG (NECK SEAM TO HEM); 83; 84; 85; 86; 87; 88; 89; 89; 1; 1. SLEEVE LENGTH SHORT (NECK SEAM TO CUFF); 75.5; 76.5; 77.5; 78.5; 79.5; 80.5; 81.5; 82.5; 1; 1. SLEEVE LENGTH REGULAR (NECK SEAM TO CUFF); 78; 79; 80; 81; 82; 83; 84; 85; 1; 1. SLEEVE LENGTH LONG (NECK SEAM TO CUFF); 80.5; 81.5; 82.5; 83.5; 84.5; 85.5; 86.5; 87.5; 1; 1. SLEEVE OPENING; 31; 32; 33; 34; 35; 36; 37; 37; 1; 1. SLEEVE WIDTH AT SEAM (BICEP); 38; 39.5; 41; 42.5; 44; 45.5; 47; 46.5; 1; 2. COLLAR CIR NECK SEAM; 41.8; 44.2; 46.6; 49; 51.4; 53.8; 56.2; 54.8; 1; 2.4. COLLAR HEIGHT CENTRE BACK; 8; 8; 8; 8; 8; 8; 8; 8.5; 0.5; 0. SLEEVE PKT WIDTH; 14; 14; 14; 14; 14; 14; 14; 14.5; 0.5; 0. SLEEVE PKT LENGTH; 17; 17; 17; 17; 17; 17; 17; 17.5; 0.5; 0. SLEEVE PKT FLAP WIDTH; 15; 15; 15; 15; 15; 15; 15; 15.5; 0.5; 0. SLEEVE PKT FLAP DEPTH; 7; 7; 7; 7; 7; 7; 7; 7.25; 0.25; 0. FRONT ZIP LENGTH; 21; 21; 21; 21; 21; 21; 21; 21.25; 0.25; 0.