| BRAND | PLATATAC |
| --- | --- |
| DESCRIPTION | JACKET |
| SIZE RANGE | XS-2XL |
| MAIN FABRIC- LINING- | 30D NYLON RIPSTOP (Ref. to Page-5 Main` fabric detail) ALPHA LINING |

GARMENT DETAILS

BACK VIEW

FRONT VIEW

OUTER SHELL

70D RIPSTOP

COLLAR

ALPHA LINING

FRONT ZIP CLOSER, LONG SLEEVE

INSIDE VIEW

LINING INSIDE- FRONT PANEL, COLLAR, BACK PANEL

SHELL INSIDE – ZIPFACING C BINDING, BACK BOTTOM, UPPER SLEEVES, BACK NECK FACING TO ATTACH LABEL, HANGER LOOP

LOWER POCKET – BOTH SIDES

#3 REVERSIBLE C/E ZIPPER WITH 1.5MM PULLAR CORD AND SHRINK WRAP

CHEST POCKET ZIPPER – WLS ONLY

#3 REVERSIBLE C/E ZIPPER WITH 1.5MM PULLAR CORD AND SHRINK WRAP

CENTER FRONT ZIPPER

#5 REVERSIBLE O/E ZIPPER WITH 1.5MM PULLAR CORD AND SHRINK WRAP

SLEEVE CUFF; VELCRO ADJUSTABLE TABS WITH ELASTIC

20MM ELASTIC TAPE

20MM VELCRO HOOK 20MM VELCRO LOOP

ADJUSTABLE HEM WITH ELASTIC CORD – WRS ONLY (ATTCH TO THE BACK SEAM 5CM FROM HEM)

2.5MM ELASTIC CORD

SMALL TOGGLE ATTCH WITH SELF FAFRIC TAPE CORD STOPER

SIZE – 2.5CM (H) X 2.5CM (W)

LOGO HEATSEAL – WEARER’S LEFT BOTTOM

| FABRIC COLOR | PRINT COLOR | PANTONE NUMB |
| --- | --- | --- |
| TFP | DARK BROWN | 19-0913 TCX |
| TIGER STRIPE | GREY | 18-5206 TCX |
| MULTICAM | DARK BROWN | 19-0913 TCX |
| MC TROPIC | GREY | 18-5206 TCX |
| MC BLACK | GREY | 18-5206 TCX |
| BLACK | GREY | 18-5206 TCX |
| RANGER GREEN | GREEN | 18-0523 TCX |
| CBK | DARK BROWN | 19-0913 TCX |
| DTS | DARK BROWN | 19-0913 TCX |
| MC ALPINE | GREY (WILD DOVE) | 17-1501 TCX |

FABRIC DETAILS

- Quantum 40D Nylon Shadowbox Ripstop, with DWR, NIR Compliant.

- Multi-cam Black – Balloon 70D Nylon Ripstop, PU Coated

- Multi-cam Tropic – Patriot Lite Textured Nylon with DWR, NIR Compliant.

- TFP, Tiger Stripe – 30D Nylon Shadowbox Ripstop, with DWR

- Plain Colors – Snug Pak Paratex Lite, Lightweight, highly windproof, water repellent and breathable

STORAGE BAG HEATSEAL

SIZE – 5.3 CM (H) X 6.3 CM (W)

| FABRIC COLOR | SORAGE BAG COLOR | PRINT COLOR |
| --- | --- | --- |
| TFP | CBK | BLACK |
| TIGER STRIPE | BLACK | GREY (18-5206 TCX) |
| MULTICAM | CBK | BLACK |
| MC TROPIC | RG | BLACK |
| MC BLACK | BLACK | GREY (18-5206 TCX) |
| BLACK | BLACK | GREY (18-5206 TCX) |
| RANGER GREEN | RG | BLACK |
| CBK | CBK | BLACK |
| DTS | CBK | BLACK |
| MC ALPINE | GREY | BLACK |

WOVEN LABELS

MAIN LABEL

MAIN LABEL – ATTACHED TO BACK NECK FACING

MAIN LABEL AND HANGER LOOP

WOVEN LABEL

SHELL FABRIC HANGER LOOP

| FABRIC COLOR | LABEL COLOR |
| --- | --- |
| TFP | DARK OLIVE BASE/ BLACK FONT |
| TIGER STRIPE | DARK OLIVE BASE / BLACK FONT |
| MULTICAM | DARK OLIVE BASE / BLACK FONT |
| MC TROPIC | DARK OLIVE BASE / BLACK FONT |
| MC BLACK | WHITE BASE/ BLACK FONT |
| BLACK | WHITE BASE/ BLACK FONT |
| RANGER GREEN | DARK OLIVE BASE / BLACK FONT |
| CBK | DARK OLIVE BASE / BLACK FONT |
| TFP | DARK OLIVE BASE / BLACK FONT |
| DTS | DARK OLIVE BASE / BLACK FONT |
| MC ALPINE | WHITE BASE/ BLACK FONT |

OTHER WOVEN LABELS

CARE LABEL

POLARTEC LABEL - BLACK

| FABRIC COLOR | CARE LABEL COLOR |
| --- | --- |
| TFP | DARK OLIVE |
| TIGER STRIPE | DARK OLIVE |
| MULTICAM | DARK OLIVE |
| MC TROPIC | DARK OLIVE |
| MC BLACK | BLACK |
| BLACK | BLACK |
| RANGER GREEN | DARK OLIVE |
| CBK | DARK OLIVE |
| TFP | DARK OLIVE |
| DTS | DARK OLIVE |

CARE LABEL AND POLARTEC LABEL – ATTACHED TO BACK SEAM OF THE SIDE PANEL (WLS – 6CM FROM HEM SEAM)

DATE LABEL - PVC

POSITION

UNDER THE CARE LABEL

SWING TAGS

POLYBAG – 40CM X 48CM – PLATATAC ZIPLOCK BAG						POLYBAG STICKER – 4CM (H) X 9CM (W)

CARTON – 60CM X 40CM X 40CM – PRINTED – 5PLY

RM CHART

NOTE: HEAT STRING WRAP TUBING – BLACK FOR ALL COLOURS

ZIPPER OPEN END PART SHOULD BE GUN METAL

| FABRIC COLOR | OUTER SHELL (70D NYLON RIPSTOP) | LINING (ALPHA) | CENTER FRONT ZIP #5 REVERABLE O/E ZIP | CHEST C LOWER PKT- #3 REV C/E ZIP | FRT, CHEST PKT, LOWER PKT 1.5MM ZIP PULL CORD | SLEEVE CUFF TAB 20 MM HOOK VELCRO | SLEEVE CUFF 20MM PILE VELCRO | BOTTOM HEM 2.5MM ELASTIC CORD | BOTTOM HEM SMALL TOGGLE AND STOPER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TFP | TFP | CBK | CBK | CBK | CBK | CBK | TFP | CBK | CBK |
| TIGER STRIPE | TIGER STRIPE | BLACK | BLACK | BLACK | BLACK | BLACK | BLACK | BLACK | BLACK |
| MULTICAM | MC | KHAKI | KHAKI | KHAKI | KHAKI | KHAKI | MC | KHAKI | KHAKI |
| MC TROPIC | MC TROPIC | RG | RG | RG | RG | RG | MC TROPIC | RG | RG |
| MC BLACK | MC BLACK | BLACK | BLACK | BLACK | BLACK | BLACK | MC BLACK | BLACK | BLACK |
| BLACK | BLACK | BLACK | BLACK | BLACK | BLACK | BLACK | BLACK | BLACK | BLACK |
| RANGER GREEN | RG | RG | RG | RG | RG | RG | RG | RG | RG |
| CBK | CBK | CBK | CBK | CBK | CBK | CBK | CBK | CBK | CBK |
| DTS | CBK | CBK | CBK | CBK | CBK | CBK | CBK | CBK | CBK |
| MC ALPINE | MC ALPINE | GREY | 13-4303 TCX | 13-4303 TCX | 13-4303 TCX | 13-4303 TCX | 13-4303 TCX | 13-4303 TCX | 13-4303 TCX |

SPEC SHEET

|  | Style : PLATATAC | SILKS ALPHA JACKET NH |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SIZE RANGE- XS- 2XL |  |  |  |  |  |  |  |  |
|  | Garment Details : |  |  |  |  |  |  |  |  |  |
|  | FINISHED GARMENT MEASUREMENTS |  | XS | S | M | L | XL | 2XL |  | TOL +/- |
|  | TO FIT CHEST SIZE |  | 98-103 | 106-111 | 114-119 | 122-127 | 130-135 | 138-143 |  |  |
| A | Total Chest Circumference (Laid Flat Pit to Pit-double) |  | 112 | 119.5 | 127 | 134.5 | 142 | 149.5 |  | 1.5 |
| B | Total Hem Circumference (Straight) |  | 100 | 107.5 | 115 | 122.5 | 130 | 137.5 |  | 1.5 |
| C | Shoulder Breadth |  | 45.6 | 47.4 | 49.2 | 51 | 52.8 | 54.6 |  | 1 |
| D | 1/2 Sleeve Girth @ Armhole |  | 27.1 | 28.3 | 29.5 | 30.7 | 31.9 | 33.1 |  | 1 |
| E | Sleeve Length (Shoulder Seam to Cuff) |  | 65.5 | 67 | 68.5 | 70 | 71.5 | 73 |  | 1 |
| F | 1/2 Sleeve Opening - Relaxed |  | 9.5 | 10 | 10.5 | 11 | 11.5 | 12 |  | 0.5 |
| G | Chest Pocket Opening |  | 15 | 16 | 16 | 17 | 17 | 17 |  | 0.5 |
| H | Front Lower Pocket Opening |  | 17 | 17 | 17 | 18 | 18 | 18 |  | 0.5 |
| I | Side Panel Width @ Hem |  | 12.4 | 13.1 | 13.8 | 14.5 | 15.2 | 15.9 |  | 0.5 |
| J | Back Panel Width @ Hem |  | 39 | 42 | 45 | 48 | 51 | 54 |  | 1 |
| K | Back Length from HSP (Highest Shoulder Point) to hem |  | 75.5 | 77.5 | 79.5 | 81.5 | 83.5 | 85.5 |  | 1 |
| L | Yoke Height from Centre Back Neck to seam |  | 8.6 | 9.2 | 9.8 | 10.4 | 11 | 11.6 |  | 0.5 |
|  | Neck Width |  | 20.6 | 21.4 | 22.2 | 23 | 23.8 | 24.6 |  | 0.5 |
|  | Front Neck Drop from HSP |  | 7.8 | 8.2 | 8.6 | 9 | 9.4 | 9.8 |  | 0.5 |
|  | Back Neck Drop from HSP |  | 3 | 3 | 3 | 3 | 3 | 3 |  | 0.5 |
| M | Collar Height @front |  | 7 | 7 | 7 | 7 | 7 | 7 |  | 0.5 |
| N | Collar Height @Center back |  | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 |  | 0.5 |
| O | Collar Circumference |  | 52 | 54 | 56 | 58 | 60 | 62 |  | 1 |
|  | Front zip Length |  | 70.5 | 72 | 73.5 | 75 | 76.5 | 78 |  | 0.5 |

METHOD OF MEASUREMENTS

SILKS NH ALPHA JACKETS SLEEVE CUFF

- 20MM PILE VELCRO CUT LENGTH - 11CM FOR ALL SIZES C STARTS 3CM FROM SL SEAM

- 20MM ELASTIC; (ELASTIC TUCK 1CM UNDER 20MM VELCRO)

| SIZE | 20MM ELASTIC CUT LENGTH |
| --- | --- |
| XS | 09 CM |
| S | 10 CM |
| M | 11 CM |
| L | 12 CM |
| XL | 13 CM |
| 2XL | 14 CM |

SILKS NH ALPHA JACKET

| SIZE | ELASTIC CORD CUT LENGTH |
| --- | --- |
| XS | 102.5CM |
| S | 110 CM |
| M | 117.5 CM |
| L | 125 CM |
| XL | 132.5 CM |
| 2XL | 140 CM |

BUTTONHOLE POSITION ON HEM FACING 1.5CM FROM SEAM (BOTH BK & FRT FACING)