DETACHABLE HOOD

FRONT ZIP CLOSER, LONG SLEEVE

SHELL INSIDE

POLY TASLON,

POCKET POLY MESH (PPS/CO/HH)

OUTER SHELL

POLAR FLEECE

| BRAND | PLATATAC |
| --- | --- |
| DESCRIPTION | JACKET |
| SIZE RANGE | MALE - XXS-XXXL s FEMALE – SIZE 6 TO 18 |
| MAIN FABRIC-LINING- | SOLID TPE BONDING POLAR FLEECE AND 500D CODURA(Ref. to Page-6 Main`fabricdetail) POLYESTER TASLON |

GARMENT DETAILS

FRONT VIEW

BACK VIEW

FRONT VIEW

BACK VIEW

INSIDE VIEW

LINING INSIDE-. POLY MESH – ALL POCKET BAGS, TASLON - BACK BOTTOM, SLEEVE POCKET

SHELL INSIDE - ZIP GUARD, COLLAR INSIDE, COLLAR INSIDE

CENTER FRONT ZIPPER

#5 VISLON ZIPPER O/E WITH 8MM TASLON PULLARS

CHEST POCKETS (BOTH SIDES)

#5 REV. C/E ZIPPER WITH 8MM TASLON PULLARS

POCKET POLYMESH (PPS/CO/HH) – POCKET INSIDE

SLEEVE POCKET (BOTH SIDES) AND VELCRO

#5 REV. C/E ZIPPER WITH 8MM TASLON PULLARS

50MM VELCRO LOOP (ATTACHED 4CM BELOW TO THE SLEEVE SEAM AND CENTER ALIGNED TO SHOULDER LINE)

POLY TASLON – POCKET INSIDE

LOWER POCKETS (BOTH SIDES)

#5 REV. C/E ZIPPER WITH 8MM TASLON PULLARS

POCKET POLYMESH (PPS/CO/HH) – POCKET INSIDE

BOTTOM HEM AND TOGGLE ATTACHMENT (BOTH SIDE ATTACH TO SIDE SEAM 5CM FROM HEM)

MEDIUM TOGGLE AND STOPPER

5MM LINING FABRIC TAPE – TOGGLE ATTCH.

2.8MM ELASTIC CORD

UNDERARM ZIPPER – MALE JACKETS ONLY

#5 REV C/E ZIPPER WITH 8MM TASLON PULLAR

OUTER NECK (BACK) – VELCRO ATTACHMENTS

20MM VELCRO LOOP AND HOOK

ADJUSTABLE HOOD WITH ELASTIC CORD - BACK

MEDIUM TOGGLE WITH STOPPER

2.8MM ELASTIC CORD

TOGGLE ATTCH – 5MM TASLON TAPE

ADJUSTABLE HOOD WITH ELASTIC CORD – SIDE TOGGLE ATTCHMNTS

MEDIUM TOGGLE ATTACHED WITH TASLON TAPE

2.8MM ELASTIC CORD CORD END (A022)

ADJUSTABLE HOOD WITH ELASTIC CORD – BOTTOM TABS WITH VELCRO

POLY TASLON TABS

20MM VELCRO LOOP AND HOOK

HOOD DETACHMENT ZIPPER

#5 REV O/E ZIPPER WITH 8MM TASLON ZIPPER PULLAR

SHOULDER EPAULATE AND VELCRO ATTACHMENTS

25MM VELCRO LOOP AND HOOK

ADJUSTABLE SLEEVE CUFF WITH VELCRO

20MM VELCRO HOOK

25MM VELCRO LOOP

## PRINT DETAILS – FRONT PANEL – WLS

PRINT SIZE – 65MM (W) X 60MM (H)

PRINT COLOR – WHITE

## FEMALE JACKETS	MALE JACKETS

| GARMENT SIZE | PLACEMENT FROM HSP (CENTER ALIGNED) |
| --- | --- |
| SIZE 6 | 16CM |
| SIZE 8 | 16CM |
| SIZE 10 | 17CM |
| SIZE 12 | 17CM |
| SIZE 14 | 17CM |
| SIZE 16 | 18CM |
| SIZE 18 | 18CM |

| GARMENT SIZE | PLACEMENT FROM HSP (CENTER ALIGNED) |
| --- | --- |
| SMALL | 19CM |
| MEDIUM | 20CM |
| LARGE | 20CM |
| XLARGE | 20CM |
| 2XLARGE | 21CM |
| 3XLARGE | 21CM |
| 4XLARGE | 21CM |

## MAIN LABEL AND HANGER LOOP

SHELL FABRIC HANGER LOOP

WOVEN LABEL – WHITE BASE (SIZE 70MM X 74MM)

## MALE LABEL	FEMALE LABEL

## RM CHART

| PLACEMENT | RM DESCRIPTION | RM COLOR 01 |
| --- | --- | --- |
| OUTERSHELL FABRIC | 98/2% POLAR FLEECE | NAVY |
| LINING FABRIC | POLY TASLON | NAVY |
| POCKET LINING | POCKET POLYMESH (PPS/CO/HH) | BLACK |
| CENTER FRONT ZIPPER | #5 VISLON O/E ZIPPER | YKK 560 |
| CHEST PKT, SLEEVE PKT, LOWER FRONT PKT, UNDERARM PKT | #5 NYLON REVERSABLE ZIPPER | YKK 560 |
| HOOD ZIPPER | #5 NYLON REVERSABLE O/E ZIPPER | YKK 560 |
| CHEST PKT, SLEEVE PKT, LOWER FRONT PKT, UNDERARM PKT C HOOD ZIPPER PULLAR TAPE | SLIDERS WITH 8MM TASLON ZIPPER PULLAR | NAVY |
| SLEEVE CUFF TAB | 20MM VELCRO HOOK | NAVY |
| SLEEVE CUFF HEM | 25MM VELCRO LOOP | NAVY |
| SLEEVE POCKET VELCRO | 50MM VELCRO LOOP | NAVY |
| SHOULDER EPAULETTE | 25MM VELCRO LOOP AND HOOK | NAVY |
| HOOD C , BOTTOM HEM – ELASTIC STRING | 2.8MM ELASTIC STRING | BLACK |
| HOOD C , BOTTOM HEM – TOGGLE C STOPPER | MEDIUM TOGGLE AND STOPPER, A022 | BLACK |
| HOOD VELCRO ATTACHING | 20MM VELCRO HOOK, 20MM VELCRO LOOP | NAVY |

| NECK CENTER BACK AND SIDE VELCRO ATTACHMENT | 20MM VELCRO LOOP AND HOOK | NAVY |
| --- | --- | --- |
| TOGGLE ATTACHMENT | 8MM TASLON TAPE | NAVY |

## FABRIC DETAILS

- MAIN FABRIC - LJFY06161-9 PD+WR (C6)+TPE+BONDING POLAR FLEECE 150G 100D+40D*100D+40D 92% POLYESTER 8% SPANDEX 310GSM CUT 54/55"

- TRIM FABRIC 01 - LJFY06257-9 320D N/TASLON PLAIN FD (LINING) PD+WR+PA 70D*320D 100% POLYESTER CUT 55/56"

- TRIM FABRIC 02 – POCKET POLYMESH – 100% POLYESTER MESH – PPS/CO/HH

## PACKING MATERIALS

HANG TAG AND POLYBAG STICKER (40MM X 100MM)	HANG TAG – LARGE PLATATAC HANG TAG

POLYBAG – 56CM X 56CM + 6CM (F) – UNPRINTED – DUST FREE – 2 AIR HOLES CARTON – 55CM X 54CM X 34CM – PRINTED – 5 PLY – K/L

| Garment Specification - TOPS 21/09/2023 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STYLE NO: | HARRY HARD 1.2 JACKET MALE |  |  | CUSTOMER: |  | Platatac |  |  |  |  |  |
| FABRIC: | SHELL (POLYESTER/SPANDEX) ,MESH |  |  | SIZE RANGE: |  | XS-4XL |  | BASE SIZE: |  |  | L |
| BLOCK: | DIGITISED PATTERN |  |  |  |  |  |  |  |  |  |  |
| GARMENT DETAILS: |  |  |  |  |  |  |  |  |  |  |  |
| FINISHED GARMENT MEASURMENTS |  | XS | S | M | L | XL | 2XL | 3XL | 4XL | TOL-/+ | GRADE |
| 1/2CHEST CIRCUM. @ ARMHOLE |  | 50 | 55 | 60 | 65 | 70 | 75 | 80 | 85 | 1 | 5 |
| 1/2HEM CIRCUMFERENCE |  | 46.5 | 50 | 55 | 60 | 65 | 70 | 75 | 80 | 1 | 5 |
| 1/2WAIST @SIDE PKT |  | 46 | 50 | 55 | 60 | 65 | 70 | 75 | 80 | 1 | 5 |
| FRONT WIDTH @HEM |  | 14.5 | 16 | 17.5 | 19 | 20.5 | 22 | 23.5 | 25 | 0.5 | 1.5 |
| CENTRE BACK LENGTH |  | 75.5 | 77 | 78.5 | 80 | 81.5 | 83 | 84.5 | 86 | 1 | 1.5 |
| FRONT LENGTH FROM HSP / FOLD |  | 69 | 70.5 | 72 | 73.5 | 75 | 76.5 | 78 | 79.5 | 1 | 1.5 |
| ARMHOLE |  | 26 | 27.5 | 29 | 30.5 | 32 | 33.5 | 35 | 36.5 | 0.5 | 1.5 |
| SLEEVE LENGTH |  | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 1 | 1 |
| SLEEVE TAB LENGTH |  | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 0.3 |  |
| UNDERARM ZIP LENGTH |  | 25 | 25 | 30 | 30 | 30 | 30 | 30 | 30 | 0.5 |  |
| 1/2BICEP @15CM FROM SHOULDER |  | 22.75 | 23.75 | 24.75 | 25.75 | 26.75 | 27.75 | 28.75 | 29.75 | 1 | 1 |
| CUFF HEIGHT |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 0.25 |  |
| 1/2SLEEVE OPENING |  | 13 | 13.5 | 14 | 14.5 | 15 | 15.5 | 16 | 16.5 | 0.5 | 0.5 |
| NECK WIDTH - EDGE TO EDGE |  | 20.2 | 20.8 | 21.4 | 22 | 22.6 | 23.2 | 23.8 | 24.4 | 0.5 | 0.6 |
| FRONT NECK DROP |  | 9.6 | 9.9 | 10.2 | 10.5 | 10.8 | 11.1 | 11.4 | 11.7 | 0.3 | 0.3 |
| BACK NECK DROP |  | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 0.3 | 0 |
| COLLAR HEIGHT @ CENTRE BACK |  | 9 | 9 | 9 | 10 | 10 | 10 | 10 | 10 | 0.5 |  |
| NECK CIRCUMFERENCE |  | 50.8 | 52.3 | 53.8 | 55.3 | 56.8 | 58.3 | 59.8 | 61.3 | 1 | 1.5 |
| CHEST PKT OPENING LENGTH |  | 22 | 22 | 22 | 22 | 22 | 23 | 23 | 23 | 0.5 |  |
| CHEST PKT POSITION FROM NECK SEAM |  | 9.5 | 9.5 | 9.5 | 9.5 | 9.5 | 9.5 | 9.5 | 9.5 | 0.5 | 0 |
| SIDE PANEL @BOTTOM |  | 17 | 17.5 | 19.5 | 21.5 | 23.5 | 25.5 | 27.5 | 29.5 | 0.5 | 2 |
| SIDE PANEL @ UNDERARM |  | 15.5 | 17 | 19 | 21 | 23 | 25 | 27 | 29 | 0.5 | 2 |
| SIDE PKT LENGTH |  | 17 | 17 | 18 | 18 | 18 | 19 | 19 | 19 | 0.5 |  |
| SIDE PKT DISTANCE FROM HEM |  | 9.5 | 10 | 10.5 | 11 | 11.5 | 12 | 12.5 | 13 | 0.5 | 0.5 |
| ACROSS SHOULDER |  | 46.5 | 48.5 | 50.5 | 52.5 | 54.5 | 56.5 | 58.5 | 60.5 | 1 | 2 |
| CF TO CB DIFFERENCE (TAIL) |  | 9.4 | 9.5 | 9.6 | 9.7 | 9.8 | 9.9 | 10 | 10.1 | 0.5 | 0.1 |
| STORM FLAP POSITION FROM HEM |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0.25 | 0 |
| FRONT ACROSS |  | 41.3 | 44.3 | 47.3 | 50.3 | 53.3 | 56.3 | 59.3 | 62.3 | 1 | 3 |
| BACK ACROSS |  | 41.3 | 44.3 | 47.3 | 50.3 | 53.3 | 56.3 | 59.3 | 62.5 | 1 | 3 |
| 1/2HOOD WIDTH @ELASTC TUNNEL |  | 24.05 | 24.7 | 25.35 | 26 | 26.65 | 27.3 | 27.95 | 28.6 | 0.5 | 0.65 |
| 1/2HOOD HEIGHT @OPENING |  | 34 | 34.5 | 35 | 35.5 | 36 | 36.5 | 37 | 37.5 | 0.5 | 0.5 |

21/09/20233:47 PM

# HARRY HARD 1.2 JACKET (MALE) ZIP LENGTH	03/05/2023

| Ref. code | MEASUREMENT POINT | Tol +/- | XS | S | M | L | XL | 2XL | 3XL | 4XL |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | INTERNAL CHEST POCKET |  |  |  |  |  |  |  |  |  |  |  |
|  | CHEST PKT OPENING finished length | 0.5 | 22.0 | 22.0 | 22.0 | 22.0 | 22.0 | 23.0 | 23.0 | 23.0 |  |  |
|  | CHEST PKT SLIDE FASTENER cut length | 0.5 | 27.0 | 27.0 | 27.0 | 27.0 | 27.0 | 28.0 | 28.0 | 28.0 |  |  |
|  | CF SLIDE FASTENER length | 0.5 | 64.5 | 65.5 | 66.5 | 68.8 | 70.0 | 71.0 | 72.0 | 73.0 |  |  |
|  | SLEEVE |  |  |  |  |  |  |  |  |  |  |  |
|  | SLEEVE PKT SLIDE FASTENER finish length | 0.5 | 16.0 | 16.0 | 16.0 | 16.0 | 16.0 | 16.0 | 16.0 | 16.0 |  |  |
|  | SLEEVE PKT SLIDE FASTENER cut length | 0.5 | 18.0 | 18.0 | 18.0 | 18.0 | 18.0 | 18.0 | 18.0 | 18.0 |  |  |
|  | UNDERARM SLIDE FASTENER finish length | 0.5 | 25.0 | 25.0 | 30.0 | 30.0 | 30.0 | 30.0 | 30.0 | 30.0 |  |  |
|  | SIDE POCKET |  |  |  |  |  |  |  |  |  |  |  |
|  | SIDE PKT OPENING finished length | 0.5 | 17.0 | 17.0 | 18.0 | 18.0 | 18.0 | 19.0 | 19.0 | 19.0 |  |  |
|  | SIDE PKT SLIDE FASTENER cut length | 0.5 | 22.0 | 22.0 | 23.0 | 23.0 | 23.0 | 24.0 | 24.0 | 24.0 |  |  |

Note: All units are in centimetres

| MGarment Specification - TOPS 28/06/2023 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STYLE NO: | HARRY HARD 1.2 JACKET FEMALE |  |  | CUSTOMER: |  | Platatac |  |  |  |  |  |
| FABRIC: | SHELL (POLYESTER/SPANDEX) ,MESH |  |  | SIZE RANGE: |  | 6-20 |  | BASE SIZE: |  |  | 12 |
| BLOCK: | DIGITISED PATTERN |  |  |  |  |  |  |  |  |  |  |
| GARMENT DETAILS: |  |  |  |  |  |  |  |  |  |  |  |
| FINISHED GARMENT MEASURMENTS |  | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | TOL-/+ | GRADE |
| 1/2 CHEST CIRCUM. @ ARMHOLE |  | 48.5 | 51 | 53.5 | 56 | 58.5 | 61 | 63.5 | 66 | 1 | 2.5 |
| 1/2HEM CIRCUMFERENCE |  | 51 | 53.5 | 56 | 58.5 | 61 | 63.5 | 66 | 68.5 | 1 | 2.5 |
| 1/2WAIST @SIDE PKT |  | 43.5 | 46 | 48.5 | 51 | 53.5 | 56 | 58.5 | 61 | 1 | 2.5 |
| FRONT WIDTH @HEM |  | 15.6 | 16.3 | 17 | 17.7 | 18.4 | 19.1 | 19.8 | 20.5 | 0.5 | 0.7 |
| CENTRE BACK LENGTH |  | 67.5 | 69.5 | 71.5 | 73.5 | 75.5 | 77.5 | 79.5 | 84 | 1 | 2 |
| FRONT LENGTH FROM HSP / FOLD |  | 64.5 | 66.5 | 68.5 | 70.5 | 72.5 | 74.5 | 76.5 | 80 | 1 | 2 |
| 1/2 ARMHOLE CIRCUMFERENCE |  | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 0.5 | 1 |
| SLEEVE LENGTH |  | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 1 | 1 |
| SLEEVE TAB LENGTH |  | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 0.25 | 0 |
| 1/2 BICEP @ 15CM FROM SHOULDER |  | 21.75 | 22 | 22.25 | 22.5 | 22.75 | 23 | 23.25 | 23.5 | 0.5 | 0.25 |
| CUFF HIGHT |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 0.25 |  |
| 1/2 SLEEVE OPENING |  | 12.95 | 13.2 | 13.45 | 13.7 | 13.95 | 14.2 | 14.45 | 14.7 | 0.5 | 0.25 |
| NECK WIDTH - EDGE TO EDGE |  | 18.9 | 19.5 | 20.1 | 20.7 | 21.3 | 21.9 | 22.5 | 23.1 | 0.5 | 0.6 |
| FRONT NECK DROP |  | 6.7 | 7 | 7.3 | 7.6 | 7.9 | 8.2 | 8.5 | 8.8 | 0.25 | 0.3 |
| BACK NECK DROP |  | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 0.25 | 0 |
| COLLAR HEIGHT @ CENTRE BACK |  | 8 | 8 | 8 | 9 | 9 | 9 | 9 | 9 | 0.5 |  |
| TOTAL NECK CIRCUMFERENCE |  | 44.5 | 46 | 47.5 | 49 | 50.5 | 52 | 53.5 | 55 | 1 | 1.5 |
| CHEST PKT OPENING LENGTH |  | 19 | 19 | 20 | 20 | 20 | 21 | 21 | 21 | 0.5 |  |
| CHEST PKT POSITION FROM NECK SEAM |  | 8.5 | 8.5 | 9.5 | 9.5 | 9.5 | 9.5 | 9.5 | 9.5 | 0.5 |  |
| SIDE PANEL @BOTTOM |  | 20.6 | 21.6 | 22.6 | 23.6 | 24.6 | 25.6 | 26.6 | 27.6 | 0.5 | 1 |
| SIDE PANEL @ UNDERARM |  | 16.2 | 16.8 | 17.4 | 18 | 18.6 | 19.2 | 19.8 | 20.4 | 0.5 | 0.6 |
| SIDE PKT LENGTH |  | 17 | 17 | 18 | 18 | 18 | 19 | 19 | 19 | 0.5 |  |
| ACROSS SHOULDER |  | 44.4 | 45.6 | 46.8 | 48 | 49.2 | 50.4 | 51.6 | 52.8 | 1 | 1.2 |
| CF TO CB DIFFERENCE (TAIL) |  | 5.5 | 5.5 | 5.5 | 5.5 | 5.5 | 5.5 | 5.5 | 5.5 | 0.5 | 0 |
| STORM FLAP POSITION FROM HEM |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0.25 | 0 |
| STORM FLAP POSITION FROM COLLAR NECK SEAM |  | 4.5 | 4.5 | 4.5 | 5 | 5 | 5 | 5 | 5 | 0.5 |  |
| FRONT ACROSS |  | 37.5 | 39 | 40.5 | 42 | 43.5 | 45 | 46.5 | 48 | 1 | 1.5 |
| BACK ACROSS |  | 39.6 | 41 | 42.4 | 43.8 | 45.2 | 46.6 | 48 | 49.4 | 1 | 1.4 |
| 1/2HOOD WIDTH @ELASTC TUNNEL |  | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 25 | 0.5 |  |
| 1/2HOOD HEIGHT @OPENING |  | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 35 | 0.5 |  |

21/09/20233:48 P

# HARRY HARD 1.2 JACKET (FEMALE) ZIP LENGTH	2/O5/2023

| Ref. code | MEASUREMENT POINT | Tol +/- | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | INTERNAL CHEST POCKET |  |  |  |  |  |  |  |  |  |  |  |
|  | CHEST PKT OPENING length | 0.5 | 19.0 | 19.0 | 20.0 | 20.0 | 20.0 | 21.0 | 21.0 | 21.0 |  |  |
|  | CHEST PKT SLIDE FASTENER cut length | 0.5 | 24.0 | 24.0 | 25.0 | 25.0 | 25.0 | 26.0 | 26.0 | 26.0 |  |  |
|  | CF SLIDE FASTENER length | 0.5 | 62.3 | 64.0 | 65.7 | 67.9 | 69.6 | 71.3 | 73.0 | 74.7 |  |  |
|  | SLEEVE |  |  |  |  |  |  |  |  |  |  |  |
|  | SLEEVE PKT SLIDE FASTENER length | 0.5 | 16.0 | 16.0 | 16.0 | 16.0 | 16.0 | 16.0 | 16.0 | 16.0 |  |  |
|  | SLEEVE PKT SLIDE FASTENER cut length | 0.5 | 18.0 | 18.0 | 18.0 | 18.0 | 18.0 | 18.0 | 18.0 | 18.0 |  |  |
|  | SIDE POCKET |  |  |  |  |  |  |  |  |  |  |  |
|  | SIDE PKT OPENING length | 0.5 | 17.0 | 17.0 | 18.0 | 18.0 | 18.0 | 19.0 | 19.0 | 19.0 |  |  |
|  | SIDE PKT SLIDE FASTENER cut length | 0.5 | 22.0 | 22.0 | 23.0 | 23.0 | 23.0 | 24.0 | 24.0 | 24.0 |  |  |

Note: All units are in centimetres

EPAULETTE (SHOULDER) IMPORTANT FINISH MEASUREMENTS FOR;

28/09/2023

PARA, MICA & AMB HARRY HARD 1.2 JACKETS

NOTE; FOR ALL SIZES MALE & FEMALE

4.5CM

1CM

13.5CM

5CM