| BRAND | PLATATAC |
| --- | --- |
| DESCRIPTION | JACKET |
| SIZE RANGE | XSMALL – 2XLARGE |
| MAIN FABRIC- LINING | GRID FLEECE (Fabric details on pg.07) POLYESTERCOTTON |

GARMENT DETAILS

FRONT

BACK

ADJUSTABLE HOOD WITH ELASTIC CORD

OUTER BODY – GRID FLEECE

FRONT ZIPPER, LONG SLEEVES

FRONT

BACK

SHELL INSIDE – FRONT ZIP CLOSER

LINING INSIDE – CHEST POCKETBAG, LOWER POCKET BAGS, BOTTOM HEM, NECK, HOOD

INSIDE VIEW

LOWER SIDE POCKETS (BOTH SIDES)

#3 REVERSIBLE C/E ZIPPER WITH

1.5MM PULLAR CORD

# CENTER FRONT ZIPPER

#5 REVERSIBLE O/E ZIPPER WITH 1.5MM PULLAR CORD

# CHEST POCKET (WLS ONLY)

#3 REVERSIBLE C/E ZIPPER WITH

1.5MM PULLAR CORD

SLEEVE CUFF WITH THUMB HOLE

BAR TACKS

# ADJUSTABLE HOOD WITH ELASTIC CORD

TUNNEL – LINING FABRIC (DTM TO SHELL)

2.8MM ELASTIC CORD SMALL TOGGLE

# ADJUSTABLE BOTTOM HEM WITH ELASTIC CORD

SMALLTOGGLE AND STOPPER (ATTACHED TO THE WLS SEAM WITH LINING FABRIC) – 5CM FROM BOTTOM HEM SEAM

2.8MM ELASTIC CORD

# MAIN LABEL AND HANGER LOOP

WOVEN LABEL – KHAKI BASE BLACK LETTERS – 7CM(W) * 6CM(H)

HANGER LOOP - LINING FABRIC

# CARE LABEL – WOVEN LABEL KHAKI BASE WITH BLACK LETTERS

SIZE - 3.5CM (W) * 4.5CM (H)

POSITION – 8CM FROM BOTTOM HEM SEAM (WLS)

# PACKING MATERIALS

PLATATAC ZIPLOCK POLYBAG (40CM X 48CM)	POLYBAG AND HANG TAG STICKER

PLATATAC LARGE HANGTAG WITH CORD	CARTON – 60*40*40CM – 5PLY – K/L - PRINTED

# RM CHART

| FABRIC COLOR | BODY GRID FLEECE | LINING POLYESTER COTTON | CF ZIPPER #5 REVERSIBLE O/E ZIPPER | POCKET ZIPPERS #3 REVERSIBLE C/E ZIPPER | 2.8MM ELASTIC CORD,1.5MM PULLAR CORD, SMALL TOGGLE AND STOPPER |
| --- | --- | --- | --- | --- | --- |
| OLIVE | OLIVE | DTM TO THE SHELL | YKK 096 | YKK 096 | RANGER GREEN |

# FABRIC DESCRIPTION

- MAIN FABRIC - 100% POLY CHECK FLEECE 250GSM +-10GSM 58"

- LINING – 95% POLYESTER, 5% COTTON – WIDTH 120CM

# SPEC SHEET