| BRAND | PLATATAC |
| --- | --- |
| DESCRIPTION | JACKET |
| SIZE RANGE | 2XS to 3XL (SHORT, REG, LONG) |
| MAIN FABRIC- LINING- | NYCO RIPSTOP POCKET POLY MESH (PPS/CO/HH) |

UNDERARM, BACK YORK AND CHEST POCKET GUSSET

POCKET POLYMESH (PPS/CP/HH)

OUTER SHELL

NYCO RIPSTOP

FRONT ZIPPER CLOSER, LONG SLEEVE

BACK VIEW

FRONT VIEW

GARMENT DETAILS

INNER VIEW

LINING INSIDE- POLYMESH – COMPASS POCKET AND FRONT POCKET GUSSET, UNDER ARM GUSSET, BACK INNER YORK.

SHELL INSIDE - ZIP GUARD, COLLAR INSIDE, SLEEVE HEM AND HANGER LOOP

CENTER FRONT ZIPPER

#5 VISLON O/E ZIP WITH 1.5MM PULLAR CORD

UNDERARM GUSSET

POCKET POLYMESH (PPS/CO/HH)

BUTTON ATTACHMENT – FRONT ZIPPER PLACKET

34L BUTTON 20MM NYLON TAPE

SLEEVE POCKET ZIPPER

#3 NORMAL C/E ZIPPER WITH 1.5MM PULLAR CORD

COMPASS POCKET

25MM VELCRO HOOK

3.5MM PARA CORD (WITHOUT CORE) – pocket inside

25MM VELCO LOOP

POCKET POLYMESH (PPS/CO/HH)

OUTER BACK YORK

POCKET POLYMESH (PPS/CO/HH)

CHEST POCKET

25MM VELCRO HOOK

3.5MM PARA CORD (WITHOUT CORE)

25MM VELCRO LOOP

34L CUT BUTTON ATTACHED WITH 20MM NYLON TAPE

CHEST POCKET GUSSET

POCKET POLYMESH (PPS/CO/HH)

SLEEVE POCKET (BOTH SLEEVES)

50MM VECLRO LOOP

25MM VELCRO HOOK

3.5MM PARA CORD (WITHOUT CORE)

25MM VELCRO LOOP

38MM VELCRO LOOP

ADJUSTABLE SLEEVE CUFF WITH BUTTON

34L CUT BUTTON ATTACHED WITH 20MM NYLON TAPE

MAIN LABEL AND HANGER LOOP

WOVEN LABEL (KHAKI BASE) – ATTACHED TO CB NECK SEAM

SHELL FABRIC HANGER LOOP

DATE LABEL PLACEMENT

UNDER THE MAIN LABEL

PVC DATE LABEL

PACKING MATERIALS

HANG TAG AND POLYBAG STICKER (40MM X 90MM)				                  PLATATAC LARGE HANG TAG

POLYBAG – PLATATAC ZIPLOCK BAG – 40CM X 48CM

CARTON – STANDARD CARTON – 60CM X 40CM X 40CM – K/L – PRINTED – 5 PLY

RM CHART

| FABRIC COLOR | OUTER SHELL NYCO RIPSTOP | TRIM FABRIC POCKET POLYMESH | FRONT ZIPPER #5 VISLON O/E ZIP | SLEEVE POCKET ZIP #3 NORMAL C/E ZIP | SLEEVE POCKET 50MM VELCRO LOOP AND 38MM VELCRO LOOP | FRONT PKT, COMPASS PKT AND SLEEVE PKT 25MM VELCRO LOOP AND HOOK | PARA CORD (WITHOUT CORE) AND 1.5MM PULLAR CORD | BUTTON ATTCH. 34L CUT BUTTON AND 20MM NYLON TAPE | THREAD (TKT 120 AND 075) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MULTICAM | MULTICAM | KHAKI | YKK 367 | YKK 367 | MULTICAM | KHAKI | KHAKI | KHAKI | 9CB20 |
| WOODLAND | WOODLAND | RG | YKK 096 | YKK 096 | RG | RG | RG | RG | 32297 |
| TIGER STRIPE | TIGER STRIPE | BLACK | YKK 580 | YKK 580 | BLACK | BLACK | BLACK | BLACK | AEBLK |
| RANGER GREEN | RANGER GREEN | RG | YKK 096 | YKK 096 | RG | RG | RG | RG | 32297 |
| MC TROPIC | MC TROPIC | RG | YKK 096 | YKK 096 | RG | RG | RG | RG | 32297 |
| DESERT TIGER STRIPE | DESERT TIGER STRIPE | TAN | YKK 573 | YKK 573 | CBK | CBK | CBK | CBK | P1505 |
| FROG SKIN | FROG SKIN | RG | YKK 096 | YKK 096 | RG | RG | RG | RG | 9CB20 |

FABRIC DESCRIPTION

- MAIN FABRIC – NYCO RIPSTOP – 50% NYLON AND 50% COTTON

- TRIM FABRIC – 100% POLYESTER – PPS/CO/HH

SPEC SHEET

METHOD OF MEASUREMENT