BACK VIEW

| BRAND | PLATATAC |
| --- | --- |
| SIZE RANGE | SMALL - 4XLARGE |
| CATEGORY | OUTER WEAR |
| DESCRIPTION | WATERPROOF JACKET |
| MAIN FABRIC | 35% NYLON, 5% SPANDEX (DWR TREATED), LINING(COLLAR); POLARTEC |

GARMENT DETAILS;

FRONT VIEW

Zippers: all zipper sliders to have fabric pullers

Front pockets: large storage pockets with water-proof zip closures

Centre front closure: waterproof zipper with 2-way sliders, covered by storm flap with Velcro and snap button (hem) closures

Collar: Polar tec lined with elastic drawstring and barrel lock adjustment Hanger loop: internal at back neck

Hem: adjustable with elastic drawstring and barrel lock

Hood with dual adjustment:

- Back (crown) adjustment with elastic cord and barrel lock

- Front (face opening) adjustment with elastic cord and barrel lock

Upper arm pockets (both sides): with water-proof zippers and Velcro loop for patches

Sleeve hems:

adjustable with Velcro tab closures

Underarm pit zips: with water proof zippers with dual sliders for ventilation control

All pockets with drainage holes for effective water drainage

Front Inside

All seams sealed to ensure waterproof performance

Back Inside

Internal zip pocket

(Wearer’s Left side only)

# FABRIC :

SHELL:

SPEC : 35% NYLON, 5% SPANDEX

WEIGHT: 175 GS/M

FINISH: PD, WD, BONDING

POLARTEC:

Inner collar & Storm ffap top

# TRIMS: DTM (Dye to match) COLOUR

| MATERIALS | DISCRIPTION | PLACEMENT |
| --- | --- | --- |
|  | 10OMM VELCRO | ON SLEEVE POCKET |
|  | 20MM VELCRO HOOK PILE | SLEEVE CUFF |
|  | BARREL-LOCK | HOOD, COLLAR s HEM |
|  | CORD END | ALL ELASTIC CORD ENDS |

| METIRIALS | DISCRIPTION | PLACEMENT |
| --- | --- | --- |
|  | CORD STOPPER | HOOD BACK CROWN |
|  | METAL SNAP BUTTON | CENTER FRONT BOTTOM HEM |
|  | EYELET (METAL) | HOOD FACING, OUTER TOP COLLAR, INNER BOTTOM COLLAR, FRONT INNER PANEL & HEMFACING |
|  | 3MM ELASTIC CORD | HOOD (FRONT & BACK) COLLAR HEM |
|  | 10MM GROSS GRAIN TAPE | FOR ALL BARRE LOCK ATTACHMENT |

| METIRIALS | DISCRIPTION | PLACEMENT |
| --- | --- | --- |
|  | YKK #5MM WATERPROOF ZIPS (REVERSE) | CENTER FRONT- Slider X 2 FRONT POCKETs - Slider X 1 UNDERARM- Slider X 2 SLEEVE POCKET- Slider X 1 |
|  | YKK #5MM REVERSE ZIP (REVERSE) | INNER POCKET |
|  | SEAM SEALING TAP | ALL INSIDE SEAMS |
|  | SELF FABRIC ZIPPER-PULLER | ATTACH TO ALL ZIP SLIDERS |
|  | 2MM CORD | HANG TAG CORD |

MAIN LABEL:

HEAT TRANSFER

COLOR: WHITE s RED

(RED PANTONE-18-1763 TCX)

PLACEMENT-

BACK NECK FACKING

18-1763 TCX

4.5CM

6CM

LABELS:

4.5CM

8.7CM

12.5CM

CARE LABEL: HEAT TRANSFER COLOR: WHITE

PLACEMENT; INNER POCKET (PATCH LABEL)

8.5CM

# SWING TAG / STICKERS;

PLATATAC BROWN CARD: SIZE- L 5CM X W 12CM

PLACEMENT:

ATTACH TO CENTER FRONT ZIP SLIDER

SWING TAG STICKER s POLY BAG STICKER:

L 4CM X W 7CM

POLY BAG: L 48CM X W 40CM

FRONT	BACK

POLYBAG STICKER PLACEMENT

BACK BOTTOM RIGHT CORNER (FACING RIGHT)

POSITION THE STICKER:

- 4 CM FROM THE BOTTOM EDGE

- 4 CM FROM THE SIDE EDGE

CARTON BOX: L C0CM X W 40CM X D 40CM