| BRAND | PLATATAC |
| --- | --- |
| SIZE RANGE | SMALL - 3XLARGE |
| CATEGORY | OUTER WEAR |
| DESCRIPTION | PANTS |
| MAIN FABRIC | 35% NYLON, 5% SPANDEX (DWR TREATED) |

GARMENT DETAILS;

FRONT VIEW

BACK VIEW

- WAIST BAND: Elasticated waist

- Front opening with waterproof zipper and slotted button

Pre-shaped Knee patch with dart

Elastic drawcord in waistband for adjustment with barrel locks (both sides) s its

attached with self-fabric.

  - Stopper for Elastic Cord ends

e.

- SIDE SEAM: 2-way waterproof

zippers at sides

Zip opening for access inside pants.

- FRONT POCKET: Both side with waterproof zippers & zip garage on top

(Patch pockets)

- Drainage buttonhole at bottom of pockets.

BELT LOOPS:

- Front: Two belt loops, each L-7 cm(Finish measurements) and W- 1.2 cm

- Back: Four belt loops, each L-7 cm (Finish measurements) and W- 1.2 cm

Page 4

- All zip sliders should have

self-fabric pulls tabs

of 18

    - LEG HEM: Adjustable Velcro tabs with elastic for secure fit adjustment

      - PLASTIC HOOK:

attached to the front panel hem seam

using elastic tape.

- WEATHER GUARD

alongside seam zipper

SEAM SEAL: All internal seams must be heat

sealed (taped) to ensure waterproof construction.

BACK INSIDE

FRONT INSIDE

FRONT HANGER:

10mm GROSS GRAIN TAPE X 2 PCS

(Both sides)

BACK HANGER:

10mm GROSS GRAIN TAPE X 1 PC

(Center back)

# FABRIC :

SHELL:

SPEC : 35% NYLON, 5% SPANDEX

WEIGHT: 175 GS/M

FINISH: PD, WD, BONDING

# TRIMS: DTM (Dye to match) COLOUR

|  | MATERIALS | DISCRIPTION | PLACEMENT |
| --- | --- | --- | --- |
| 1 |  | SLOTED BUTTON | ON WAIST BAND - ATTACH C SECURE WITH SELF-FABRIC TAPE. |
| 2 |  | BARREL-LOCK | FRONT WAISTBAND –ATTATCH AT THE FRONT WAISTBAND SEAM WITH SELF -FABRIC TAPE |
| 3 |  | PLASTIC BELL END STOPPER -CORD ENDS | ATTCH TO ALL ELASTIC CORD ENDS |
| 4 |  | PLASTIC HOOK | FRONT HEM SEAM– ATTATCH WITH 10MM ELASTIC |

|  | MATERIALS | DISCRIPTION | PLACEMENT |
| --- | --- | --- | --- |
| 5 |  | 10MM ELASTIC | FRONT BOTTOM HEM – HOOK ATTATCHMENT |
| 6 |  | 25MM ELASTIC | LEG TAB |
| 7 |  | 50MM ELASTIC | WAIST BAND |
| 8 |  | 3MM ELASTIC CORD | WAISTBAND - ADJUSTABLE |
| G |  | 10MM GROSS GRAIN TAPE | WAIST BAND- ATTACH AS LOOP (HANGER) |

|  | MATERIALS | DISCRIPTION | PLACEMENT |
| --- | --- | --- | --- |
| 10 |  | 20MM HOOK VELCRO 25MM PILE VELCRO | BOTTOM HEM THREAD TO VELCRO TAB BOTTOM HEM |
| 11 |  | YKK #5MM WATERPROOF ZIPS (REVERSE) | CENTER FRONT- Slider X 1 FRONT POCKETs - Slider X 1 SIDE SEAM- Slider X 2 |
| 12 |  | SELF FABRIC ZIPPER-PULLER | ATTACH TO ALL ZIP SLIDERS |
| 13 |  | SEAM SEALING TAPE | ALL INSIDE SEAMS |

| 14 |  | 2MM CORD | HANG TAG CORD |
| --- | --- | --- | --- |
| 15 |  | PIPIN CORD | SIDE SEAM (BOTH SIDES) |

# LABELS:

MAIN LABEL:

HEAT TRANSFER

COLOR: WHITE s RED

(RED PANTONE-18-1763 TCX)

CARE LABEL: HEAT TRANSFER COLOR: WHITE

# PLACEMENT ;

MAIN LABEL:

INSIDE BACK

WEARE’S RIGHT SIDE

CARE LABEL:

INSIDE BACK

WEARE’S LEFT SIDE

PLATATAC BROWN CARD: SIZE- L 5CM X W 12CM

SWING TAG / STICKERS;

SWING TAG STICKER s POLY BAG STICKER:

SIZE- L 4CM X W 7CM

PLACEMENT:

ATTACH TO THE FRONT BELT LOOP

# POLY BAG (PLATATAC):

SIZE - L 40CM X W 30CM

FRONT	BACK

POLYBAG STICKER PLACEMENT

BACK BOTTOM RIGHT CORNER (FACING RIGHT)

POSITION THE STICKER:

- 4 CM FROM THE BOTTOM EDGE

- 4 CM FROM THE SIDE EDGE

CARTON BOX LABEL- STICKER /PRINT:

CHANGE ACCORDINGLY

CARTON BOX -MARKS & NUMBERS:	SIZE - C0CM X 40CM X 40CM

LABEL - STICKER / PRINT

FRONT s BACK

SIDES